# Calibrating Thermometers

## (Sample SOP)

PURPOSE: Ensure that thermometers are regularly calibrated to maintain food safety.

SCOPE: This procedure applies to employees who receive, prepare, cook, transport, serve, cool, and reheat food.

KEY WORDS: Thermometers, Calibration, Temperature, Ice Point, Boiling Point

INSTRUCTIONS:

- Train school nutrition employees on the proper procedures for calibrating food thermometers using this SOP.

- Refer to the Calibrating Food Thermometers (Sample SOP) and Using Food Thermometers (Sample SOP).

- Follow State or local health department requirements.

- Calibrate the food thermometer using an appropriate method.

    - Ice Point method (preferred)

- Fill a cup or container with ice and add enough cold water to fill until 1 inch below the lip.

- Allow to sit for one minute.

- Insert probe fully into the ice-water slush.

- Ensure that the dimple or sensing area is completely submerged.

- Ensure that the thermometer probe is not touching the sides or bottom of the container.

- Wait 30 seconds for temperature to stabilize.

- Required reading: 32 °F (±2 °F) [0°C ±1 °C].

- If thermometer reads incorrectly, adjust per manufacturer instructions (calibration nut for bimetallic, reset button for digital) or remove from service.

    - Boiling Point method

    - Bring a pot of water to boil.

- Pot must be large enough for the thermometer dimple or sensing area to be completely submerged and not touch the sides or bottom.

    - Immerse the probe to the dimple into boiling water.

    - Allow the temperature reading to stabilize before reading temperature.

    - Reading should be 212 ºF (+ 2 ºF) [or 100 ºC (+ 1 ºC)].

- This reading may vary at higher altitudes.

    - If adjustment is required, follow manufacturer’s instructions.

- Calibrate thermometers at appropriate times.

- At the start of each operational day (strongly recommended)

- At minimum once per week

- After any drop or impact

- After measuring extreme temperatures

- Clean and sanitize food thermometers before each use.

- Refer to the Cleaning and Sanitizing Food Contact Surfaces (Sample SOP) for the proper procedure to follow.

- Store food thermometers in an area that is clean and where they are not subject to contamination or frequent jolting.

MONITORING:

  - School nutrition employees will use either the ice-point method or boiling-point method to verify the accuracy of food thermometers.

  - Schedule routine calibration of thermometers.

CORRECTIVE ACTION:

- Retrain any school nutrition employee found not following the procedures in this SOP.

- For an inaccurate, bimetallic, dial-faced thermometer, recalibrate it by adjusting the temperature by turning the dial while securing the calibration nut (located just under or below the dial) with pliers or a wrench.

- For an inaccurate, digital thermometer with a reset button, adjust the thermometer according to manufacturer’s instructions.

- If an inaccurate thermometer cannot be adjusted on-site, discontinue using it, and follow manufacturer’s instructions for having the thermometer calibrated.

- If a thermometer that is out of calibration was found to be used on a food item, check the food item with a properly calibrated thermometer and reheat food if necessary.

- Refer to the Reheating Time/Temperature Control for Safety Foods (Sample SOP) if reheating is needed.

VERIFICATION AND RECORD KEEPING:

School nutrition employees will:

- Record the calibration temperature and any corrective action taken, if applicable, on the Thermometer Calibration Log each time a thermometer is calibrated.

The school nutrition manager will:

- Verify that employees are calibrating thermometers properly by making visual observations of the employees during the calibration process and all operating hours.

- Review and initial the Thermometer Calibration Log periodically.

- Keep the Thermometer Calibration Log for a period of six months following a month's temperature records to demonstrate compliance, or as directed by your State agency.

DATE IMPLEMENTED: __________________ BY: _______________________

DATE REVIEWED: ___________________	BY: _______________________

DATE REVISED: _______________________	BY: _______________________

Source: Institute of Child Nutrition. (2026). HACCP-based standard operating procedures. University, MS: Author.