# Recovering from a Water Outage

(Sample SOP)

PURPOSE: Ensure a school nutrition kitchen is safe to use after a water outage.

SCOPE: This procedure applies to school nutrition professionals who may have to prepare food after a water outage.

KEY WORDS: Water outage, emergency preparedness, emergency recovery, potable water, disruption, boil water

INSTRUCTIONS:

- Check all equipment and facilities to ensure they are working properly when water service is restored.

- Take follow-up action in accordance with the water utility company’s guidelines and manufacturer’s instructions when water supply is safe.

- Flush plumbing systems, including faucets, toilets, and fountains, before use.

- At least 5 minutes of flushing is typically recommended for faucets

- At least 10–15 minutes is typically recommended for equipment water lines (e.g., ice machines, fountains, and beverage machines).

- Clean and sanitize food contact surfaces, utensils, and equipment before use, including those with water connections (e.g., beverage machines and dishwashers).

- Refer to the Cleaning and Disinfecting Food Contact Surfaces Sample SOP.

MONITORING:

- Follow guidance provided by the water utility company or contact the local health authority for guidance to ensure water is safe.

- Ensure that at least one trained employee per shift is designated to respond to a water outage.

- Complete the Recovery after Short-Term (< 4 hours) Water Loss with No Contamination Checklist for water outage events shorter than four hours and the Recovery Following a Long-Term Interruption of Water Service Checklist for those longer than four hours.

CORRECTIVE ACTION:

- Retrain any school nutrition employee found not following the procedures in this SOP.

- Continue to flush lines that were not flushed for the appropriate time.

- Re-clean any equipment that was done incorrectly.

- Restock cleaning supplies and personal protective equipment.

- Restock potable water supply.

- Restock disposable utensils and supplies.

VERIFICATION AND RECORD-KEEPING:

- Record end time and date of water outage on the Recovery after Short-Term (< 4 hours) Water Loss with No Contamination Checklist or the Recovery Following a Long-Term Interruption of Water Service Checklist.

- Verify that the Recovery after Short-term (< 4 hours) Water Loss with No Contamination Checklist or the Recovery Following a Long-Term Interruption of Water Service Checklist is complete. Retain for three years plus the current school year, or as directed by your State agency.

- Review and revise emergency plan to confirm or update response strategies.

DATE IMPLEMENTED: __________________ BY: ______________________

DATE REVIEWED: _____________________ BY: ______________________

DATE REVISED: _______________________ BY: ______________________

Source:

Institute of Child Nutrition. (2026). HACCP-based standard operating procedures manual. University, MS: Author.

Institute of Child Nutrition. (2021). Emergency preparedness plan: Templates. University, MS: Author

Institute of Child Nutrition. (2015). Recovery after short-term (< 4 hours) water loss with no contamination Checklist. University, MS: Author

Institute of Child Nutrition. (2015). Recovery following a long-term interruption of water service checklist. University, MS: Author

U.S. Department of Agriculture, Food and Nutrition Service, Food Safety Branch. (2022, April). Food safety emergency response pocket guide disaster edition. https://theicn.org/icn-resources-a-z/emergency-response-pocket-guide/