# Responding During Water Contamination

(Sample SOP)

PURPOSE: Ensure the availability of potable water (suitable for human consumption) during a water contamination event to ensure safe water for drinking, handwashing, food preparation, cleaning, and sanitizing.

SCOPE: This procedure applies to child nutrition professionals who may prepare and serve food during a water contamination incident.

KEY WORDS: Water Contamination, Emergency Preparedness, Emergency Response, Potable Water, Disruption, Boil Water, Bacteria, Boil Water Notice

INSTRUCTIONS:

- Immediately assess impact on your food service operation (i.e., the size of the affected area and affected equipment).

- Determine, in consultation with the health authority and emergency management official(s), if and how limited food service operations can continue.

- Food service activities should discontinue if there is an immediate health hazard.

- Coordinate with school officials to contact the water utility company to determine the cause, extent, and estimated duration of the event.

- Locate the appropriate emergency equipment and supplies needed (e.g., bottled water, bulk water storage containers, disposable utensils, gloves, hand sanitizer).

- Do not use any equipment connected to the water line, including ice machines and water fountains. Covers or signs should be posted on equipment to prevent use.

- Discard any food, beverages, and ice prepared before the advisory or incident, or that may have been exposed to contamination.

- Use only clean water from an approved source (e.g., bottled, boiled, or treated water). Do not use the affected water supply if it has not been determined safe.

- If approved source options are unavailable, only boiled water should be used for consumption and food service activities.

  - Bring water to a rolling boil for at least one minute, or three minutes at elevations above 6,500 feet.

  - Allow water to cool before use or drinking.

- Additional alternative water sources that may be used include hauled water from an approved public water supply in a covered sanitized container or a licensed potable drinking water tanker truck.

- Set up a temporary handwash station to continue handwashing. Determine if hand sanitizer is an appropriate alternative to handwashing.

- Do not handle ready-to-eat foods with your bare hands. Use gloves, utensils, and deli paper to handle food.

- Use disposable utensils and supplies until a clean and safe water supply is available for dishwashing and sanitizing.

  - Limited food service should be discontinued when these items are exhausted or unavailable.

MONITORING:

- Record start time and dates of water contamination notice on the Response to a Short-Term Loss of Water Service Checklist for those shorter than four hours and the Response to a Long-Term Loss of Water Service Checklist for water outages longer than four. Complete the checklist.

- Place appropriate signage or covers to indicate that water fountains and sinks are unsafe to drink from.

- Follow guidance in boil water notice provided by the water utility company or contact the local health authority for guidance.

- Maintain potable water supplies that are sufficient for food service operations.

- Maintain adequate disposable utensils and supplies.

CORRECTIVE ACTION:

- Retrain any school nutrition employee found not following the procedures in this SOP.

- Discard any food that may have come in contact with contaminated water and log on the Damaged or Discarded Product Log.

VERIFICATION AND RECORD-KEEPING:

- Record any food thrown away due to water contamination on the Damaged or Discarded Product Log.

- Verify that an incident report was completed and filed. Retain for three years plus the current school year, or as directed by your State agency.

- Verify that the Damaged or Discarded Product Log and the Response to a Long-Term Loss of Water Service Checklist or Response to a Short-Term Loss of Water Service Checklist are completed. Retain for three years plus the current school year, or as directed by your State agency.

DATE IMPLEMENTED: __________________ BY: ______________________

DATE REVIEWED: _____________________ BY: ______________________

DATE REVISED: _______________________ BY: ______________________

Source:

Institute of Child Nutrition. (2026). HACCP-based standard operating procedures manual. University, MS: Author.

Institute of Child Nutrition. (2021). Emergency preparedness plan: Templates. University, MS: Author

Institute of Child Nutrition. (2015). Response to a long-term loss of water service checklist. University, MS: Author

Institute of Child Nutrition. (2015). Response to a short-term loss of water service checklist. University, MS: Author

U.S. Department of Agriculture, Food and Nutrition Service, Food Safety Branch. (2022, April). Food safety emergency response pocket guide disaster edition. https://theicn.org/icn-resources-a-z/emergency-response-pocket-guide/