# Using Food Thermometers

## (Sample SOP)

PURPOSE: Ensure that accurate, appropriate thermometers are used for all temperature measurements.

SCOPE: This procedure applies to employees who receive, prepare, cook, transport, serve, cool, and reheat food.

KEY WORDS: Thermometers, Temperature, Logs, Cooking, Reheating, Holding, Transporting

INSTRUCTIONS:

- Train school nutrition employees on the proper procedures for using food thermometers using this SOP.

- Follow State or local health department requirements.

- Follow the food thermometer manufacturer’s instructions for use. In the absence of instructions, use the following guidance for thermometers:

  - Hold the face or dial of the thermometer, not the stem.

  - Insert the stem into the center of the thickest part of the food.

  - Insert thermometer into food until the dimple/notch on the stem of the thermometer is covered by the food.

  - Keep the thermometer in place in the food until the temperature stabilizes, roughly 30 seconds.

  - Read the temperature.

  - Take a second temperature reading in a second location in large pots, skillets, or pans.

  - Record the temperature on the respective log.

  - Take corrective action if needed.

- Use a food thermometer that measures temperatures from 0 °F (-18 °C) to 220 °F (104 °C) and is appropriate for the temperature being taken. For example:

- Thermistor or thermocouple with a thin probe that is tip-sensitive: takes temperatures of thin products, such as hamburgers, pizza, filets, nuggets, and sausage patties.

- Bimetallic, dial-faced stem thermometers: accurate only when measuring temperatures of thick foods. They may not be used to measure temperatures of thin foods. A dimple mark located on the stem of the thermometer indicates the maximum food thickness that can be accurately measured as this area between tip and dimple is the sensing area.

- Oven-safe thermometers: use when measuring temperatures of food while cooking in an oven.

- Infrared thermometers: takes the surface temperature and should be used for frozen foods, packaged foods, and milk cartons.

- Irreversible temperature indicator: disposable temperature measuring devices that change color when the desired temperature is achieved. They should only be used for thin items which have a temperature requirement that matches the indicator. Also appropriate for measuring dishwasher temperature.

- Take the temperature of food using the correct method for different food types.

  - Casseroles: check temperature in the center and at several other points.

  - Milk carton: open a carton and insert thermometer at least 2 inches into the milk.

  - Packaged food: use an infrared thermometer to take a surface temperature or place the thermometer between two packages without puncturing the packages.

  - Roasts: insert thermometer in the middle, avoiding any bones and cartilage.

  - Thin meats: insert thermometer into the side of food until 2–3 inches deep.

  - Whole poultry: insert thermometer at the thickest part, avoiding any bones and cartilage.

- Have food thermometers easily accessible to school nutrition employees during all hours of operation.

- Clean and sanitize food thermometers before each use.

- Refer to the Cleaning and Sanitizing Food Contact Surfaces (Sample SOP) for the proper procedure to follow.

- Store food thermometers in an area that is clean and where they are not subject to contamination or frequent jolting.

MONITORING:

  - School nutrition managers will ensure that there are enough thermometers on site to be able to set any non-functioning ones aside and still check temperatures.

  - School nutrition employees will use the appropriate thermometers to complete tasks and follow proper temperature checking procedures.

CORRECTIVE ACTION:

- Retrain any school nutrition employee found not following the procedures in this SOP.

- Retrain employees who are using food thermometers improperly or not taking temperatures correctly.

VERIFICATION AND RECORD KEEPING:

School nutrition employees will:

- Record the food temperatures on the appropriate logs for different tasks.

  - Cooking and Reheating Temperature Log

  - Cooling Temperature Log

  - Hot and Cold Holding Temperature Log

  - Production Log

  - Receiving Log

  - Thawing Foods Log

The school nutrition manager will:

- Verify that employees are using thermometers properly by making visual observations of the employees during the calibration process and all operating hours.

- Review and initial the temperature logs used.

- Complete the Food Safety Checklist daily.

- Keep the temperature logs for a period of six months following a month's temperature records to demonstrate compliance, or as directed by your State agency.

- Keep Food Safety Checklist for three years plus the current school year, or as directed by your State agency.

# DATE IMPLEMENTED: __________________ BY: ______________________

DATE REVIEWED: _____________________ BY: ______________________

DATE REVISED: _______________________ BY: ______________________

Source:

Institute of Child Nutrition. (2026). HACCP-based standard operating procedures manual. University, MS: Author.